# State of Israel

# Ministry of Transport and Road Safety

# Department of Motor Vehicles and

# Maintenance Services

Israeli Mandatory Requirements Safety and Environmental Protection Regulations For Motor Vehicles Manufactured According To European Union Directives For the Calendar Year 2026

# Introduction

The Israeli Mandatory Requirements (IMR) for motor vehicles manufactured according to the European Union directives is the statutory document for importing vehicles into the State of Israel.

This IMR document is for informational purposes only. The document includes information for all vehicle categories as well as the options for importing vehicles in compliance with the updated standards and regulations of the EC member states and with additional Israeli requirements.

In general, vehicles manufactured in accordance with European Community safety and environmental directives and regulations, as required by EC countries for new type of vehicles and for new vehicles (and for vehicles for "New Registrations"*) through the IMR calendar year 2026 are approved for import into the State of Israel according to the Type of Date as requested in the EC countries.

* New Registrations date: is the date after which all new vehicles being sold and first registered must meet the new or revised requirements (i.e. the date by which all existing models must have been updated to meet the new or revised requirements).

In addition, the vehicle must comply with Israeli's Road Traffic Ordinance and Regulations.

Vehicle registration approval may be carried out through the following options:

- WVTA framework

- Separate directive framework (applies to specific pinpointed vehicle categories)

The manufacturer must be familiar with the requirements for the specific vehicle category intended for import and proceed according to the instructions listed in the applicable "Registration Requirements" document.

Import of vehicles must be carried out by an accredited importer, holding an import license issued by the Israeli Ministry of Transport and Road Safety (MOT).

The accredited importer must submit the completed Israeli "Registration Requirements" document signed by the manufacturer and duly witnessed by a notary.

Documentation confirming that the vehicle was manufactured according to EC directives and regulations and permitted for use on public roads in the EC member states must be submitted as well. In certain cases, and for certain items, the manufacturer will be requested to provide a copy of additional documentation/certification.

All certificates and documents required by the IMR must be submitted to the MOT in English.

The MOT reserves the right to update the requirements for motor vehicle during the calendar year 2026.

The "Registration Requirements" document form can be downloaded from the MOT website at https://www.gov.il/he/departments/policies/imr_rr_m_n_o_2026.

# Chapter 1

# Vehicle Categories

Vehicle categories are defined in Israel according to the following classifications:

Category M: 	Motor vehicles with at least four wheels designed and constructed for the carriage of passengers.

M1:	Vehicles designed and constructed for the carriage of passengers, comprising not more than eight seats in addition to the driver's seating position and having a maximum mass not exceeding 3.5 tonnes. Vehicles belonging to Category M1 shall have no space for standing passengers. The number of seating positions may be restricted to one (i.e. the driver’s seating position).

M2:	Vehicles designed and constructed for the carriage of passengers, comprising more than eight seats in addition to the driver's seat, and having a maximum mass not exceeding 5 tonnes. Vehicles belonging to Category M2 may have space for standing passengers in addition to the seating positions.

M3:	Vehicles designed and constructed for the carriage of passengers, comprising more than eight seats in addition to the driver's seat, and having a maximum mass exceeding 5 tonnes. Vehicles belonging to Category M3 may have space for standing passengers.

Category N:	Motor vehicles with at least four wheels designed and constructed for the carriage of goods.

N1:	Vehicles designed and constructed for the carriage of goods and having a maximum mass not exceeding 3.5 tonnes.

N2:	Vehicles designed and constructed for the carriage of goods and having a maximum mass exceeding 3.5 tonnes but not exceeding 12 tonnes.

N3:	Vehicles designed and constructed for the carriage of goods and having a maximum mass exceeding 12 tonnes.

Category	O:	Trailers (including semi-trailers).

O1:	Trailers with a maximum mass not exceeding 0.75 tonnes.

O2:	Trailers with a maximum mass exceeding 0.75 tonnes but not exceeding 3.5 tonnes.

O3:	Trailers with a maximum mass exceeding 3.5 tonnes but not exceeding 10 tonnes.

O4:	Trailers with a maximum mass exceeding 10 tonnes.

Chapter 2

# Information Document for the Purpose of Vehicle Registration in Israel

For the purpose of vehicle registration in Israel, a comprehensive list of vehicle information according to vehicle category must be supplied by the manufacturer.

See Chapter 2 of the "Registration Requirements" document, for the information requested.

# Chapter 3

# Vehicle Registration Requirements

This chapter establishes the requirements for vehicles, trailers, systems, components and separate technical units intended for vehicle registration and entry into service in Israel.

The chapter is divided into three different sections as follows:

  - Registration requirements for vehicles produced under the Whole Vehicle Type Approval (WVTA) framework and documentation.

  - Registration requirements for vehicles produced under the Separate Directives and Regulations framework and documentation.

  - Additional vehicle requirements according to Israel's Road Traffic Ordinance and Regulations.

Vehicle registration in Israel may be carried out through the following options:

- WVTA framework – the application must contain the documentation required by section 3.1 and 3.3.

The WVTA framework applies to vehicle for which the enforcement date is obligatory according to section 3.1.

The WVTA framework is optional for other vehicle categories, according to the manufacturer's decision.

- Separate directive framework - the application must contain the documentation required by section 3.2 and 3.3.

The separate directives framework can be applied to specific vehicles as listed in section 3.2.

  - Whole Vehicle Type Approval (WVTA) Framework

From 01.09.2020 Framework Regulation (EU) 2018/858 replaced Framework Directive 2007/46/EC and subsequent amendments for all new type-approvals.

Type Approvals according Framework Directive 2007/46/EC and subsequent amendments granted before 01.09.2020 will not be invalidated and may still be extended if revised in accordance to article 33 or 34 of Regulation (EU) 2018/858.

    - Directive 2007/46/EC and Regulation (EU) 2018/858 (WVTA) and subsequent amendments established a framework for approval of all new motor vehicles and their trailers. The Frameworks apply to vehicles designed and constructed in one or more stages for on-road use.

    - The Frameworks contain administrative provisions and general technical requirements for approval of all new vehicles within their scope.

    - The MOT has adopted the 'EC type approval' for vehicles produced in unlimited series according to Directive 2007/46/EC and the 'EU type approval' according to Regulation (EU) 2018/858 from 01.09.2020 as the main frameworks for approval.

    - Enforcement dates for the implementation of Directive 2007/46/EC and Regulation (EU) 2018/858 are mandatory for various categories, as marked in the 'WVTA Requirements' table presented below.

    - Consequently, the MOT will permit registration and entry into service in Israel of vehicles as follows:

- Vehicles in categories M, N and O produced in unlimited series

  - complete and incomplete vehicles

This paragraph details the regulatory requirements of complete and incomplete vehicles at the completion of the first stage

  - The vehicles must:

      - Comply with the 'EC type approval' according to Directive 2007/46/EC or 'EU type approval' according to Framework Regulation (EU) 2018/858.

      - Meet the technical requirements 'list of regulatory acts for EC type-approval of vehicles produced in unlimited series' specified by Annex IV part I of Directive 2007/46/EC or by Annex II part I of Framework Regulation (EU) 2018/858. An alternative ECE regulation will be accepted in accordance with part II in Annex IV of 2007/46/EC or part II in Annex II of Regulation (EU) 2018/858.

      - Have an 'EC type–approval' certificate granted in accordance with Directive 2007/46/EC or 'EU type–approval' certificate granted in accordance with Framework Regulation (EU) 2018/858.

      - Is of the type enabling the manufacturer to issue a Certificate of Conformity (COC) for the vehicle for registration in EC member states.

  - In addition, the manufacturer must provide the most recent 'EC type–approval'/ 'EU type–approval' certificate (the most recent extension number) in use by EC members and valid in EC countries.

  - completed vehicles

# This paragraph details the regulatory requirements of completed vehicles.

The vehicles must:

- For their first stage, comply with the requirements listed above in paragraph 'A.1'.

- For the construction made at the second and subsequent stages, meet the technical requirements 'list of regulatory acts for EC type-approval of vehicles produced in unlimited series' specified by Annex IV part I of Directive 2007/46/EC or by Annex II Part I of Regulation (EU) 2018/858.

- Have an 'EC type–approval' certificate for the completed vehicle granted in accordance with Directive 2007/46/EC or Framework Regulation (EU) 2018/858.

- Vehicles in categories N2 and N3 produced in small series

  - At completion of the first stage, the vehicle must:

      - Comply with 'National type approval' and have a 'type–approval' certificate granted according to article 23 of Directive 2007/46/EC or to article 42 of Framework Regulation (EU) 2018/858.

      - Meet the technical requirements 'list of regulatory acts for EC type-approval of vehicles produced in unlimited series' specified by Annex IV part I of Directive 2007/46/EC or by Annex II Part I of Regulation (EU) 2018/858.

      - The provided 'type–approval' certificate is valid in one of the EC countries.

      - The vehicle has a statutory document issued by an approval authority or other official authority through which it may be registered in one of the EC member states.

  - In addition, the manufacturer must provide the most recent 'EC type–approval' certificate (the most recent extension number) in use by EC members and valid in EC countries.

- Vehicles produced pursuant to articles 23-25 of Directive 2007/46/EC or to articles 42-47 of Framework Regulation (EU) 2018/858.

Each vehicle category must have documentation confirming that the vehicle is approved according to Directive 2007/46/EC or Regulation (EU) 2018/858. In addition, vehicles must meet requirements as follows:

  - Special Purpose Vehicle (SPV):

  - Comply with the 'EC type approval' and have an 'EC type–approval' certificate issued in accordance with Directive 2007/46/EC or Regulation (EU) 2018/858.

  - Meet the technical requirements specified by Annex IV or Annex XI of Directive 2007/46/EC or Article 44 of Regulation (EU) 2018/858.

  - Vehicles in category N2, N3 and O constructed or modified at the second and subsequent stages:

This section refers to vehicles that:

- Were constructed or modified at the second and subsequent stages pursuant to articles 24, 25 of Directive 2007/46/EC or to articles 44-47 of Regulation (EU) 2018/858.

- Are within the permitted Gross Vehicle Weight (GVW) as defined in the 'body builder instructions' or as defined on the approval certificate of the first stage homologation.

- Are within the permitted requirements as defined in Israeli's Road Traffic Ordinance and Regulations.

The vehicles must:

  - For their first stage, comply with the requirements listed above in paragraph 'A1' or 'B'.

  - Have an 'individual approval' certificate confirming that the vehicle was approved according to Directive 2007/46/EC or Regulation (EU) 2018/858.

Or

Have a certificate issued by a technical service, confirming that the separate technical units manufactured, modified or added were constructed according to the manufacturers 'body builder instructions'.

Or

Have a certificate issued by the first stage manufacturer confirming that the body builder is approved by him and that the separate technical units manufactured, modified or added were constructed according to his instructions.

Or

Have two certificates as follows:

Self-certificate issued by the body builder and certificate issued by the manufacturer's representative in Israel, confirming that the separate technical units manufactured, modified or added were constructed according to the manufacturers 'body builder instructions'.

  - The MOT may request the manufacturer to submit a statutory document, issued by an approval authority or other official authority through which they may be registered in one the EC member states.

  - Vehicles in categories N and O with abnormal/exceptional weight/load or dimension:

This section refers to vehicles in categories N and O built in one stage or constructed/modified at the second and subsequent stages, that are associated under the 'abnormal/exceptional weight/load or dimension' ("tziud harig") configurations as defined in Israeli's Road Traffic Ordinance and Regulations.

The vehicles must:

- If constructed in more than one stage, the first stage must comply with the requirements listed above in paragraphs 'A.1' or 'B'.

- Have a certificate issued by a technical service, confirming that the separate technical units manufactured, modified or added were constructed according to the manufacturer's 'body builder instructions'. Alternatively, the MOT will accept submission of the certificate issued by the first stage manufacturer.

- Have an information document regarding the maneuverability of the vehicle, the permitted axel loads, etc.

- Have a document issued by an approval authority, technical service or other official authority stating that the vehicle may be registered in one of the EC member states.

  - Vehicles in categories O

The vehicles must have a document, issued by an approval authority, technical service or other official authority stating that the vehicle may be registered in one of the EC member states.

  - Irregular vehicles in categories M and N:

This section refers to vehicles:

- in category M and N constructed in one stage or constructed in second and subsequent stages, not in accordance with the original manufacturer instructions;

- associated to a specific configuration as follows:

- Vehicles in category M that could not be approved and certified according to paragraph 'A.1'. (For example: limousines, special vehicles produced for government use, etc.)

- Vehicles in category N that couldn't be approved and certified according to paragraph 'C.2' or 'C.3'. (For example: special armored vehicles, special cranes, amphibious vehicles, etc.)

Note:

Registration of vehicles listed in this paragraph will be considered only in cases where the necessity for the specific configuration is proven beyond any doubt. The MOT will examine the applications through a professional committee subject to internal procedures.

The vehicles must have a document, issued by an approval authority, technical service or other official authority stating that the vehicle may be registered in one the EC member states.

- Vehicles in categories M1, N1 produced in small series pursuant to article 22 of Directive 2007/46/EC or to article 41 of Regulation (EU) 2018/858

This section established a framework for import of vehicle in a quantity of not more than 10 vehicles per model (type/WVTA), per IMR calendar year.

The vehicles must meet the technical requirements listed in article 22 of Directive 2007/46/EC or article 41 of Regulation (EU) 2018/858.

Note:

The MOT reserves the right to expand or specify additional configurations if it becomes apparent that the manufacturer cannot provide the documentation as required by IMR or by Israeli's Road Traffic Ordinance and Regulations.

The MOT reserves the right to request additional documentation as needed.

# Whole Vehicle Type Approval (WVTA) Requirements

| Subject | Directive / Regulation |  | Vehicle Category |  |  |  |  |  |  |  |  |  | Enforcement/ Implementation Dates |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Base | As Amended By | M1 | M2 | M3 | N1 | N2 | N3 | O1 | O2 | O3 | O4 |  |
| Type Approval of Motor Vehicles and Their Trailers | 2007/46/EC | (EC) 661/2009 / (EU) 2019/2144 | X | X | X | X | X | X | X | X | X | X | Mandatory for TA granted before 01.09.2020 & as long not withdrawn & Pursuant to new or revised requirements |
|  | (EU) 2018/858 | (EU) 2019/2144 (EU) 2020/683 & (EU) 2021/535 – (for new TA from July 2022) & (EU) 2022/2236 – (for new TA from July 2024) | X | X | X | X | X | X | X | X | X | X | (EU) 2018/858: mandatory for new TA from 09.2020 (in accordance with the relevant implementation dates) (EU) 2019/2144 - GSR: mandatory in accordance with the relevant implementation dates |
|  |  | (EU) 2021/1958 Intelligent Speed Assistance (ISA) for new Registrations | X | X | X | X | X | X |  |  |  |  | The import of new vehicles will be approved only if the vehicle manufacturer declares that the vehicles have ISA system adapted to Israel, as stated within “MOT'S DECISION REGARDING THE ISA-IMPLEMETATION IN ISRAEL” Ref. no.: 4000-0402-2023-000279 |
| Electric & Plug-in hybrid electrical vehicles (PHEV) | Electric & Plug-in hybrid electrical vehicles (PHEV) must comply with the requirements listed under section 3.3. |  |  |  |  |  |  |  |  |  |  |  |  |
| Remarks: - In certain cases, the MOT will consider approving registration according to Directive 70/156/EEC and amendments of vehicles in category M1 whose registration or entry into service in the EC countries were prior to April 29, 2009. The manufacture must present a COC and to verify that their entry into service in the EC countries is not prohibited according to EC regulations. - Buses must meet additional requirements as stated in the "Israeli Mandatory Requirements for Buses" |  |  |  |  |  |  |  |  |  |  |  |  |  |

  - Separate Directives and Regulations framework

- This section is derived from Directive 2007/46/EC or Regulation (EU) 2018/858 and contains administrative provisions, general technical requirements for approval, registration and licensing of the follows vehicles only:

- Locally manufactured vehicles.

- Vehicles manufactured for new technology testing or operational concept analysis.

- Vehicles manufactured for evaluation purposes.

- Armored vehicles.

- Special needs vehicles.

- Etc.

- Vehicles approved under this section must comply with the directives and regulations listed in Directive 2007/46/EC Annex IV part I or in Regulation (EU) 2018/858 annex II part I 'list of regulatory acts for EC type-approval of vehicles produced in unlimited series' or with the directives and regulations listed in Directive 2007/46/EC annex XI or Regulation (EU) 2018/858 annex IV part III.

- In light of the singularity of vehicles under this section, the MOT will examine each vehicle in question according to EC regulation requirements and according to Israeli administrative provisions.

- Vehicle manufacturers must submit technical service certification for each separate directive and regulation required.

- Separate directive and regulation requirements for the systems and components in this section must be included in the comprehensive prototype inspection.

- The manufacturer must present additional verification that his obligations regarding the conformity of production (COP) remain valid throughout the calendar year 2026.

- Motor vehicle manufacturers may attach ECE documentation as alternative to the EC regulation requirements. The alternative ECE documentation must comply EC Member State requirements and with regulations set out in each separate directive as listed in Annex IV of Directive 2007/46/EC or Annex II parts I and II of Regulation (EU) 2018/858.

Note:

The manufacturer may be required to submit certification from an approval authority or technical service regarding the ECE regulation.

- A vehicle that complies with the IMR must be able to be registered in EC member states. The MOT reserves the right to request a statement from the approval authority or technical service laboratory attesting to this.

  - Additional Requirements According to Israeli Road Traffic Ordinance and Regulations

Documentation for each directive, regulation and standard in this section must comply with the required implementation date and latest subsequent amendments/edition.

These separate requirements are in addition to the requirements in section 3.1 or 3.2 and should be submitted for the components, systems and engine (combustion, electric) installed in the vehicle.

|  | Subject | Directive / Regulation |  | Vehicle Category |  |  |  |  |  |  |  |  |  | Remarks |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Base | As Amended By | M1 | M2 | M3 | N1 | N2 | N3 | O1 | O2 | O3 | O4 |  |
| 3.3.1 | Recording Equipment in Road Transport (Tachograph) | 3821/85/EEC 165/2014/EEC (EU) 2016/799 | (EU) 2016/130 (EU) 2018/502 |  |  | X |  | X | X |  | The MOT intends to implement the Digital Tachograph Regulation during the first quarter of 2026, which will be followed by an official announcement. - Vehicles equipped with analog tachographs may be ordered and will be accepted for registration only if the order was placed prior to the official announcement and the registration is completed no later than three months after the official announcement. - Orders placed after the announcement date: only vehicles equipped with a Digital Tachograph Gen 1 will be accepted for registration, regardless of the delivery date. Importers are strongly advised to plan procurement in accordance with these requirements to avoid registration delays or non-compliance. |  |  | - According to Israeli Road Traffic Ordinance and Regulations: - Installation of recording equipment is mandatory on buses and commercial vehicles exceeding 8,000 kg GVW - The recording equipment must be calibrated. |
| 3.3.2 | Flammability of Interior Materials or Burning Behavior of materials used in the Interior Construction | (EC) 661/2009 OR (EU) 2019/2144 (from July 2022) OR ECE-118 OR FMVSS 302 | (EU) 2019/543 | X | X | X | X | X | X |  |  |  |  | This requirement was expanded to include vehicles in Categories M (all class), N. Vehicles in these categories will be accepted according to: A. manufacturer's self-certificate. OR B. Israeli Std. 373 and 400 requiring a flammability test according to ISO 3795 as follows: - For all interior materials for buses, up to 100 mm/min. - For seat cushions and covers for all vehicles, up to 100 mm/min. - For other interior materials in driver and passenger compartment, up to 250 mm/min |
| 3.3.3 | Spare Wheel and Tire Unit and Run Flat Tires | ECE 64 |  | X |  |  | X |  |  |  |  |  |  | The requirement for Spare Wheel is according to Israeli Road Traffic Ordinance and Regulations – paragraphs 365 & 365 (A) |
| 3.3.4 | Tank Vehicle Rollover Stability | ECE 111 |  |  |  |  |  | X | X |  |  | X | X | For ADR trucks and tankers only |

|  | Subject | Directive / Regulation |  | Vehicle Category |  |  |  |  |  |  |  |  |  | Remarks |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Base | As Amended By | M1 | M2 | M3 | N1 | N2 | N3 | O1 | O2 | O3 | O4 |  |
| 3.3.5 | Fuel Type -Unleaded Gasoline | Israeli Std. 90 Part 2 |  | X | X | X | X | X | X |  |  |  |  |  |
| 3.3.6 | Fuel Type -Diesel Engines | Israeli Std. 107 Part 1 |  | X | X | X | X | X | X |  |  |  |  |  |
| 3.3.7 | Air Conditioner | (EU) 2018/858 & Israeli Road Traffic Ordinance and Regulations 364 (c) |  | X | X | X | X | X | X |  |  |  |  |  |
| 3.3.8 | Electrical Connectors of towing And Towed Vehicles (15 Pole) | ISO 12098 |  |  |  |  |  | X | X |  |  | X | X | Applies to vehicles and trailers equipped with coupling device |
|  | Electrical Connectors of towing And Towed Vehicles (13 Pole) | ISO 11446 |  | X | X |  | X |  |  | X | X |  |  |  |
| 3.3.9 | A.B.S. Electrical Connection | ISO 7638 |  |  |  |  |  | X | X |  |  | X | X | - Applicable for vehicles not equipped with EBS - The vehicles should be equipped with a voltage socket for 24v according to MOT requirement (No. 59004 Dated July 2005) |

|  | Subject | Directive / Regulation |  | Vehicle Category |  |  |  |  |  |  |  |  |  | Remarks |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Base | As Amended By | M1 | M2 | M3 | N1 | N2 | N3 | O1 | O2 | O3 | O4 |  |
| 3.3.10 | Buses – Additional requirement as follow: | Israeli Mandatory Requirements for Buses (IMRFB) 4000-0402-2025-000182 | June. 2017 addition |  | X | X |  |  | X |  |  |  |  | 1) Category N3 is for locally manufactured special purpose vehicles 2) Additional requirements may be required by the Public Transport Authority department of MOT for buses used for public transportation. |
|  | Specifications and Performance |  |  |  |  |  |  |  |  |  |  |  |  | See IMRFB paragraph 3.1 |
|  | Air conditioning |  |  |  |  |  |  |  |  |  |  |  |  | See IMRFB paragraph 3.2 |
|  | Vehicle Interior (steps, Luggage racks and compartments) |  |  |  |  |  |  |  |  |  |  |  |  | See IMRFB paragraph 3.3 |
|  | Vehicle Exterior (service doors, bus roof, driver's compartment, drivers' field of vision) |  |  |  |  |  |  |  |  |  |  |  |  | See IMRFB paragraph 3.4 |
|  | Driver's compartment |  |  |  |  |  |  |  |  |  |  |  |  | See IMRFB paragraph 3.5 |
|  | Driver's field of vision |  |  |  |  |  |  |  |  |  |  |  |  | See IMRFB paragraph 3.6 |
|  | Interior mirrors |  |  |  |  |  |  |  |  |  |  |  |  | See IMRFB paragraph 3.7 |
|  | Litter Bins |  |  |  |  |  |  |  |  |  |  |  |  | See IMRFB paragraph 3.8 |
|  | Automatic Fire Extinguisher |  |  |  |  |  |  |  |  |  |  |  |  | See IMRFB paragraph 3.9 |

|  | Subject | Directive / Regulation |  | Vehicle Category |  |  |  |  |  |  |  |  |  | Remarks |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Base | As Amended By | M1 | M2 | M3 | N1 | N2 | N3 | O1 | O2 | O3 | O4 |  |
| 3.3.11 | Rear Window Defroster | Israeli Road Traffic Ordinance and Regulations – paragraph 357 (A) |  | X |  |  |  |  |  |  |  |  |  | Manufacturer's certificate will be accepted. |
| 3.3.12 | Head Restraints of Seats | Israeli Road Traffic Ordinance and Regulations – paragraph 355 (c) |  |  |  |  | X | X |  |  |  |  |  |  |
| 3.3.13 | Safety Chains | Israeli Road Traffic Ordinance and Regulations – paragraph 375 |  |  |  |  |  |  |  | X | X | X | X |  |
| 3.3.14 | Fenders (Wings) |  |  |  |  |  |  |  |  | X | X |  |  | Fenders (wings) required if wheels extend out of the trailer body |
| 3.3.15 | Automotive Air Brake Line Couplers | SAE J318 OR ISO 1728 |  |  |  |  |  | X | X |  |  | X | X | Applies to automotive air brake line couplers used to connect the brake system of vehicles and trailer |
| 3.3.16 | Reversing - Buzzer | ECE 165 OR SAE J994 OR JSAE JASO D901 |  |  | X | X |  |  | X |  |  |  | X | According to Israeli Road Traffic Ordinance and Regulations – paragraph 359 (A) – reversing buzzer applies to: - Buses - Commercial vehicles exceeding 15,000 kg - Construction vehicles (cranes, pumps, Etc.) - Trailers and semi-trailers exceeding 15,000 kg GVW |
| 3.3.17 | Vehicle identification number (VIN) | ISO 3779 | (EU) 2019/2144 2021/535 | X | X | X | X | X | X | X | X | X | X |  |
| 3.3.18 | Devices to Prevent Unauthorized Use (Anti-Theft, Immobilizer and Alarm System) | 74/61/EEC OR | 95/56/EC | X | X | X | X | X | X |  |  |  |  | In compliance with the Israeli regulations on the prevention of noise, published in Israel's Official Gazette "Reshumot" on January 27, 2011, the installation of audible alarm devices in new vehicles is prohibited |
|  |  | ECE 116 |  | X |  |  | X |  |  |  |  |  |  |  |
|  |  | ECE 18 |  |  | X | X |  | X | X |  |  |  |  |  |

|  | Subject | Directive / Regulation |  | Vehicle Category |  |  |  |  |  |  |  |  |  | Remarks |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Base | As Amended By | M1 | M2 | M3 | N1 | N2 | N3 | O1 | O2 | O3 | O4 |  |
| 3.3.19 | Motor Vehicles and Trailers Intended for The Transport of Dangerous Goods. | (EC) 661/2009 OR ECE 105 | (EU) 2019/2144 (from July 2022) |  |  |  | X | X | X |  |  | X | X | - This regulation does not replace other mandatory requirements. - Each vehicle should have EC Type-Approval and type approval certificate or EC Type Approval certificate according to the definitions in Directive 2007/46/EC or Regulation (EU) 2018/858. - Incomplete, complete and completed vehicles intended to be adapted in Israel for the transportation of dangerous goods must comply with this directive as well. |
| 3.3.20 | Automotive natural gas (CNG) - quality Specification | Israeli Std. 6119 |  | X | X | X | X | X | X |  |  |  |  |  |
| 3.3.21 | Speed limiter | Israeli Road Traffic Ordinance and Regulations – paragraph 364 e |  |  |  | X |  | X | X |  |  |  |  | Applies to vehicle categories M3, N2 and N3 exceeding 7,500 Kg G.V.W. |

| Electrical & Plug-in Hybrid vehicles In certain cases, the MOT will examine, through a professional committee, giving waivers to electric vehicles produced in limited series, in accordance with the EC countries policy, for implementation of this new technology. The MOT reserves the right to update the mandatory requirements for electric vehicles during the calendar year 2026. The electric charging requirements and connecting vehicle to the charging station are subject to Israeli relevant standards. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Subject | Directive / Regulation |  | Vehicle Category |  |  |  |  |  |  |  |  |  | Remarks |
|  |  | Base | As Amended By | M1 | M2 | M3 | N1 | N2 | N3 | O1 | O2 | O3 | O4 |  |
| 3.3.22 | Electrical cord extension set - used for the connection between the vehicle and the charging station | Israeli standard 61851-1 |  | X | X | X | X | X | X |  |  |  |  | Document confirming that the electrical cord is compatible with the electrical vehicle charging system and with the charging station must be submitted by the manufacturer or as an alternative by the 'Standards Institution of Israel'. |
| 3.3.23 | External Identification markings (in addition to the markings displayed by the manufacturer) |  |  | X | X | X | X | X | X |  |  |  |  | The vehicle must display 3 external markings, making it easily identifiable as an electric vehicle: - One on the back of the vehicle; - One on each side of the vehicle on the inlet covers. If there is only one inlet, the second marking is to be displayed in the corresponding position on the other side. Note: The exact position of the identification markings must be approved by the MOT. The Markings could be done in Israel. |

# Chapter 4

| Declaration |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | I hereby declare that the given information is true and correct. |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Name |  |  |  |  |  | Position |  |  |  |  |  |  |
|  | At (Name of Manufacturer) |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Date: |  |  |  |  |  |  |  |  | Signature |  |  |  |
|  | Notary: I the undersigned |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Public notary at |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Hereby certify that |  |  |  |  |  |  |  |  |  |  |  | is authorized |
|  | to sign declarations in the context of vehicle Type Approval and vehicle registration in Israel on behalf of the manufacturer. |  |  |  |  |  |  |  |  |  |  |  |  |
|  | This person appeared before me and signed on (Date) |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Person’s identity no. / passport no. is |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | I have here into set my seal and signature |  |  |  |  |  |  |  |  |  |  |  |
|  |  | Date |  |  |  |  |  | Signature |  |  |  |  |  |

Note:

If the status as a representative of the manufacturer has not been changed, a declaration from previous years duly witnessed by a notary will be accepted.

| # Chapter 5 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Description of Vehicle Identification Number (VIN) - Content and structure (according to standard ISO 3779); from July 2022 according to Regulation (EU) 2021/535 for new Type Approvals) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 | 17 |  |
| Note: A copy of the VIN description / VIN code explanation / VIN structure used by the manufacturer can be attached. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |